Guys,

there is some basic difference. Bob and John are talking about using the

buit-in Linux driver (aka. kernel module ftdi_sio) I guess.

The xlxd code makes use of the closed-source driver ft2xx by FTDI.

いくつかの基本的な違いがあります。 ボブとジョンは、

buit-in Linuxドライバ（別名カーネルモジュールftdi_sio）だと思います。

xlxdコードは、FTDIによってクローズドソースドライバft2xxを使用します。

We are currently struggeling with resetting the ThumbDV correctly.

私たちは現在、ThumbDVを正しくリセットすることに苦労しています。

vy73 de Florian DF2ET

On 08/12/2017 04:37 AM, Yngwie Chou yngwie.chou@gmail.com [xlxd-star] wrote:

> Thanks John.

>

> Initiating USB-3012 or USB-3003 is a different story from USB-3000.

> I’ll need to introduce specific codes for USB-3000 / ThumbDV.

>

> Understood that ThumbDV uses FT230XS instead of FT232R.

> Thanks for the hints, I’ll take a look at the FT230X to see if there’s

> any necessary change required.

>

> Here the output of my current code, ambed finds the ThumbDV correctly.

> But I wasn’t able to perform a Packet Reset.

>

> root@compass:/ambed# /ambed/ambed 10.100.1.39

> Starting AMBEd

>

> Initializing vocodecs:

> Detected 2 USB-FTDI devices

>

> Description : USB-3000Serial : DN27WI1V

> Description : USB-3000Serial : DN27WI0Z

> Found USB-3000 (0) S/N: DN27WI1V

> Found another USB-3000 (1) DN27WI0Z

> Paring USB-3000 devices

> Opening USB-3000:DN27WI1V device

> Using baudrate = 460800 for USB-3000

> Setting baudrate = 460800

> Reseting USB-3000 device

> Trying soft reset for USB-3000

> [DEBUG] Sending PKT_RESET w/o Parity

> [DEBUG] ftStatus = 0

> [DEBUG] Get 0 bytes

> FTDI_read_bytes(4) failed : 0

> [DEBUG] Sending PKT_RESETSOFTCFG w/o Parity

> [DEBUG] ftStatus = 0

> [DEBUG] Get 0 bytes

> FTDI_read_bytes(4) failed : 0

> reset failure

>

> Codec interfaces initialized successfully : 0 channels availables

>

> Initializing controller

>

> AMBEd started and listening on 10.100.1.39

> ^C

> root@compass:/ambed#

>

>

> Regards,

> Yngwie

> BU2EA

>

>> On Aug 12, 2017, at 10:25, john_fields@yahoo.com

>> <mailto:john_fields@yahoo.com> [xlxd-star] <xlxd-star@yahoogroups.com

>> <mailto:xlxd-star@yahoogroups.com>> wrote:

>>

>> Hi Yngwie,

>>

>>

>> I have a USB-3012 which is recognized as 4, 3-channel devices. So 12

>> channels are seen.

>>

>> Bob/W6KD pointed out that the ThumbDV uses the FTDI FT230XS

>> serial-to-USB chip. The DVSI units use the FT232R, but both use the

>> same unified driver set. Apparently FTDI has a Facebook page and

>> describes the differences.

>>

>> Might be useful to look there and also on the FTDI site and see if any

>> differences might indicate different settings in the drivers. Just a

>> thought.

>>

>> Good luck!

>>

>> John

>> K6KD

>>

>