XLXの作者の方から回答がありましたので共有します

The development team for XLX including the AMBE code is LX1IQ and LX3JL. Any "official" information would need to come from them. I will just share what I know:

AMBEコードを含むXLXの開発チームはLX1IQとLX3JLです。 どんな "公式な"情報もそれらから来る必要があります。 私は私が知っていることを共有するだけです：

The AMBE code is there just for limited testing. It will not work with the current XLX 1.2.4. It needs XLX 2 which has not been released yet.

The AMBE code has been tested with USB-3003 and USB-3012. It may work with the ThumbDV but that has not yet been tested and it is not known if some configuration changes might be necessary.

AMBEコードは、USB-3003およびUSB-3012でテスト済みです。 それはThumbDVで動作するかもしれませんが、まだテストされておらず、設定変更が必要かもしれないかどうかは分かりません

When XLX 2 is released, you can access modules A, B, C, ... as regular D-STAR modules. They will be associated with DMR talk groups 4001, 4002, 4003 ... The actual number of modules and talk groups depend on how XLX is configured in main.h and the capacity of the transcoding devices attached to the AMBE server.

XLX 2がリリースされると、通常のD-STARモジュールとしてモジュールA、B、C、...にアクセスできます。 これらは、DMRトークグループ4001,4002,4003に関連付けられます。モジュールとトークグループの実際の数は、main.hでXLXがどのように設定されているか、またAMBEサーバに接続されているトランスコーディングデバイスの容量によって異なります

For gateways and repeaters, you might be interested in MMDVM software. With the new DMRGateway software that works with MMDVM software, multiple DMR servers can be accessed and talk groups can be remapped to that the talk groups in one system do not conflict with the talk groups of another system. You can get help on this from the Yahoo Group mmdvm. There are also images for the RPi that contain this software. An alternative gateway that can access multiple systems (D-STAR, DMR, Fusion) is openSPOT. I have threads here you might be interested in:

ゲートウェイとリピータの場合、MMDVMソフトウェアに興味があるかもしれません。 MMDVMソフトウェアで動作する新しいDMRGatewayソフトウェアを使用すると、複数のDMRサーバーにアクセスでき、1つのシステム内のトークグループが別のシステムのトークグループと競合しないようにトークグループを再マッピングできます。 あなたはYahoo Group mmdvmからこれに関する助けを得ることができます。 このソフトウェアを含むRPiのイメージもあります。 複数のシステム（D-STAR、DMR、Fusion）にアクセスできる代替ゲートウェイはopenSPOTです。 興味があるかもしれないスレッドがあります：

d-star-roundtable.boards.net/thread/260/opinion-comparing-kb5rab-pi-star

d-star-roundtable.boards.net/thread/260/opinion-comparing-kb5rab-pi-star

d-star-roundtable.boards.net/thread/261/comparing-mmdvm-dmrgateway-openspot

These threads can also be reached at roundtable.tech in the Current Discussions folder.

73

John

K6KD