chan_dstar.c is the Asterisk channel driver for D-Star.

This driver requires a dongle to convert analog to digital audio

and back.

You will also need to download the dextra_cli_active package for connecting

to D-STAR/dextra_srv, D-STAR XRF reflectors

or download the dplus_client package for connecting to D-STAR/dplus, D-STAR REF reflectors.

For people using SIP phones, now

using a SIP phone and making use of the

asterisk channel driver, chan_dstar.c contained in this

package, can connect to D-STAR repeaters

and reflectors.

You do not have to make any changes on the D-STAR repeater/gateway

or the D-STAR reflector.

The chan_dstar.c driver on the client user machine

will handle  the connection from the SIP phone thru asterisk

to the remote D-STAR system.

The steps to make it work are as follows:

1)

copy chan_dstar.c to the /usr/src/asterisk/channels directory

or wherever you have the channels subdirectory of the asterisk source tree.

2)

rebuld the Asterisk server with

gmake

gmake install

3)

In /etc/asterisk/rpt.conf

define a node that has the rxchannel set to this:

rxchannel=dstar/0.0.0.0:9990:7770

4)

In /etc/asterisk/modules.conf, make sure you load chan_dstar.so

just in case you have set autoload=no

5)

If you want to connect to D-STAR/dextra_srv nodes or D-STAR XRF reflectors,

configure the dextra_cli package

by looking at dextra_cli_conf.txt and dextra_cli_connect.txt

If you want to connect to D-STAR/dplus nodes or D-STAR REF reflectors

configure the dplus_client package by

looking at dplus_client_conf.txt and dplus_client_connect.txt

so that they match the rxchannel=dstar/0.0.0.0:9990:7770 in /etc/asterisk/rpt.conf

7)

Read the file dextra_cli_readme.txt  or the dplus_client_readme.txt

The instructions in that file tell you how to set the dongle up.

8)

Link in your Asterisk radio node to the driver node that has

rxchannel=dstar/0.0.0.0:9990:7770

or just attach the SIP phone to the asterisk node that has

rxchannel=dstar/0.0.0.0:9990:7770

(Use a DTMF command to link the 2 nodes)

Your Asterisk radio node usually has this:

rxchannel = Radio/usb

9)

Start the Asterisk server and then plug-in the d-star dongle.

10)

Start the  dextra_cli_active software if you want to connect to D-STAR/dextra_srv or D-STAR XRF reflectors

or start the dplus_client software if you want to connect to D-STAR/dplus or D-STAR REF reflectors.

73,

KI4LKF